Project Design Document

1. Design Intent

What experience are you trying to create?

A working scale-model elevator that a user can control wirelessly from their phone. The system feels responsive and realistic — floors are selected on a custom-built app, the cabin travels the correct distance, doors open automatically on arrival confirmed by an IR sensor, a floor LED lights up, and a dedicated emergency button in the app brings the cabin safely to ground at any time.

Who is it for?

A student and maker audience demonstrating embedded systems concepts in a single integrated prototype: Bluetooth communication, time-based DC motor control, interrupt-driven IR sensing, PWM servo actuation, and NeoPixel LED feedback — all controlled by a single ESP32 microcontroller running MicroPython.

What problem does it solve?

It demonstrates how a real elevator control system works — input command → motor travel → sensor-confirmed arrival → door actuation → standby — in a tangible, testable, low-cost prototype. The use of hardware interrupts for IR sensing (instead of polling) models how production embedded systems handle time-critical signals reliably.

2. Inspiration & References

| Reference | What was learned |
| --- | --- |
| Real elevator panels | Floor buttons send discrete single-char commands; arrival confirmed by physical sensor at each floor |
| MIT App Inventor BT | One-byte UART send is sufficient; ListPicker handles BT device scan and connect; IsConnected guard prevents crashes |
| motor driver repository | IN1/IN2 logic table for direction; ENA pin required for enable; VCC = 5 V separate from logic |
| NeoPixel repository | Single-wire protocol on 5 V; 470 Ω series resistor on DIN line is essential to prevent signal reflection |
| IR obstacle sensor modules | Output is active LOW; highly sensitive to motor vibration — must settle 2 s after motor stop before polling / interrupting |
| MicroPython Pin.irq() docs | IRQ_FALLING on input pin fires ISR on HIGH→LOW edge; ISR must be minimal (flag only) to avoid re-entrancy faults |

3. User Journey

The interaction is designed to feel like a real elevator.

Normal operation

- User powers on the system — elevator initialises at floor 0, doors closed, all LEDs off, serial prints 'Elevator ready at floor 0.'

- User opens the MIT App Inventor app and taps the ListPicker to scan for Bluetooth devices. The app populates the list from BluetoothClient1.AddressesAndNames.

- User selects the ESP32 from the list. The app calls BluetoothClient1.Connect. Connection confirmed when IsConnected returns true.

- User taps a floor button (Floor 0 / 1 / 2). The app checks IsConnected, then calls BluetoothClient1.SendText with the character '0', '1', or '2'.

- ESP32 receives the single-character UART command and calls go_to_floor(target).

- Motor runs — up or down — for the calibrated travel time (floor_ms[target] − floor_ms[current_floor] milliseconds).

- Motor stops. System waits 2 000 ms (IR_SETTLE_MS) for vibrations to die down. Only then does the IR interrupt arm.

- IR sensor at the target floor breaks its beam and fires the ISR instantly — ir_flag[n] is set. wait_for_ir() detects the flag and returns. If IR does not trigger within 3 s the fallback opens doors anyway.

- Floor LED lights up (blue-white). Both servo motors open the doors simultaneously.

- Doors hold open for 3 000 ms (DOOR_HOLD_MS).

- Floor LED turns off. Doors close. System returns to standby and waits for the next app command.

Emergency path

- User taps the red Emergency button in the app. App sends 'E' over Bluetooth.

- ESP32 receives 'E', immediately calls motor_stop() and begins emergency_sequence().

- All NeoPixel LEDs light up amber one by one (visual alarm animation).

- If not already at floor 0, motor runs down for floor_ms[current_floor] milliseconds to reach ground.

- Motor stops. System settles 2 000 ms. Doors open. Hold 3 000 ms. Doors close.

- All LEDs turn off. System resets to standby at floor 0.

4. Success Criteria

| Category | Criterion | Target |
| --- | --- | --- |
| Functional | Motor response time | Command → motor start within 200 ms of app tap |
| Functional | Floor arrival accuracy | Arrives within ±1 s of calibrated floor_ms value |
| Functional | IR confirmation | IR confirms or fallback opens doors within 3 s of motor stop |
| Functional | Door response | Doors fully open within 1 s of IR confirmation |
| Functional | Emergency response time | 'E' received → motor stop within 100 ms |
| Functional | IR interrupt latency | ISR fires within ~10 µs of beam break (no polling delay) |
| UX | App interface | One tap per floor · clearly labelled · BT status visible |
| UX | Visual feedback | Floor LED and door motion confirm arrival without verbal feedback |
| UX | Emergency accessibility | Red emergency button prominent · sends 'E' over BT · immediate response |
| Reliability | IR flakiness tolerance | 3-second fallback: doors open even if IR sensor misfires |
| Reliability | Clean reset after emergency | System returns to floor 0 standby with LEDs off every time |
| Reliability | ISR safety | ISR body is one line only (flag set) · no hardware calls inside ISR |

5. System Overview

The system is built around an ESP32 microcontroller running MicroPython. It receives floor commands — and an emergency override — wirelessly from a phone app via Bluetooth serial (UART2 on TX 17 / RX 16). A DC motor and L298N driver move the elevator cabin up and down a track, timed to pre-measured floor distances stored in floor_ms[]. Two servo motors actuate the sliding doors via PWM on GPIO 18 and 19.

Three IR sensors — one per floor on GPIO 34, 35, and 32 — confirm cabin arrival using hardware interrupts (Pin.IRQ_FALLING). Each sensor has its own ISR closure that sets ir_flag[n] the instant the beam is broken. The main code only reads and clears these flags; it never polls the raw pin value. A 2-second settle delay after the motor stops prevents vibration from triggering a false positive. A 3-second fallback timeout guarantees doors always open even if a sensor is flaky.

A NeoPixel LED strip on GPIO 23 provides visual floor indication (blue-white) and emergency alarm (amber animation). There is no physical emergency button — the emergency function is triggered entirely by the app sending the character 'E' over Bluetooth, which calls motor_stop() and the emergency_sequence() function immediately.

Main components

- ESP32 microcontroller — central controller, MicroPython firmware

- DC motor + L298N motor driver — cabin travel (GPIO 25 / 26 / 27, 5 V)

- 2× servo motors — sliding doors (GPIO 18 / 19, PWM 50 Hz, 5 V)

- 3× IR obstacle sensors — arrival confirmation (GPIO 34 / 35 / 32, 3.3 V, IRQ_FALLING)

- NeoPixel LED strip (10 LEDs) — floor indicator and emergency alarm (GPIO 23, 5 V)

- Power module — 5 V rail (motors, servos, LEDs) and 3.3 V rail (IR sensors); common GND

- MIT App Inventor phone app — user interface; sends '0' / '1' / '2' / 'E' over Bluetooth

What uses time vs. what uses interrupts

- DC motor travel is still time-based. The motor runs for floor_ms[target] − floor_ms[current_floor] milliseconds and then stops. There is no encoder or position feedback.

- IR sensor detection is interrupt-driven. Pin.IRQ_FALLING fires the ISR the instant the beam breaks. No polling loop is used.

- The 2-second IR settle delay is a deliberate time.sleep_ms() to let the cabin stop vibrating before the IR interrupt is considered valid.

- The emergency sequence uses time for the return descent (same floor_ms timing as normal travel).

Note: To make motor travel truly position-accurate, rotary encoders or mechanical limit switches at each floor would be needed. That is a hardware addition beyond the current design scope.

6. High-Level Block Diagram

See the software block diagram and electronics block diagram produced as companion visualisations. The summary flow is:

Phone app → Bluetooth → ESP32 → Motor driver → DC motor (travel)

↓

IR sensor ISR (arrival confirm)

↓

Servo motors (door open / close) + NeoPixel (floor LED)

↑

Emergency: app sends 'E' → motor stop → return to floor 0 → reset

7. Input / Output Mapping

| Input | Process (ESP32) | Output |
| --- | --- | --- |
| App btn: Floor 0 / 1 / 2 | UART decode → calculate travel_ms → run motor | DC motor rotates up or down |
| Travel timer expires | Stop motor · update current_floor · begin door cycle | Motor stops |
| Motor stopped for 2 000 ms | Clear ir_flag · IR interrupt now active on target floor | IR ISR armed |
| IR FALLING edge (beam broken) | ISR sets ir_flag[n] instantly · wait_for_ir() unblocks | LED on · servos open doors |
| Door hold 3 000 ms elapsed | LED off · close doors · standby | Servos close · NeoPixel off |
| App: Emergency button tap | Send 'E' over BT → motor_stop() → leds amber → descend to F0 | All LEDs amber · doors open · reset |

8. Sketches & Concepts — Design Iterations

v1 — Single floor, timer-only

- Two floors only (0 and 1)

- Doors opened on a fixed timer after motor stop — no sensor confirmation

- No emergency function

- Individual LEDs per floor

v2 — Three floors + NeoPixel + emergency

- Extended to three floors by adding a third value to floor_ms[]

- Replaced individual LEDs with a single NeoPixel strip for floor indication and emergency animation

- Added emergency sequence: stop motor, amber LED animation, return to floor 0, open/close doors, reset

v3 — IR sensors added (polling)

- Identified problem: doors opened on timer alone with no physical confirmation the cabin had actually arrived

- Added one IR obstacle sensor per floor; OUT pin polled every 20 ms in a while loop after motor stop

- Added 2-second settle delay after motor stop before polling begins — motor vibration was causing false triggers

- Added 3-second fallback timeout so doors always open even if the IR sensor misfires

v4 — IR interrupts replace polling

- Identified problem: tight polling loop was busy-waiting and blocked all other processing during IR wait

- Replaced Pin.value() polling with Pin.irq(trigger=Pin.IRQ_FALLING) on all three IR sensor pins

- ISR closures (one per floor via make_ir_isr factory) set ir_flag[n] = True on beam break

- wait_for_ir() now sleeps in 10 ms slices and only checks the flag — no raw pin reads

- Result: IR detection is edge-triggered and near-instantaneous; CPU is free between checks

v5 — Emergency button moved to app (final)

- Original design had a physical emergency button on GPIO 14 with its own ISR

- Decision: remove the physical button entirely. The app already has a persistent Bluetooth connection; sending 'E' is equally immediate and reduces wiring complexity

- Physical emergency_btn pin and emergency_isr removed from code

- Main loop now handles 'E' as a UART command exactly like '0' / '1' / '2'

- Emergency button in the app is styled red and larger than floor buttons to remain visually prominent

- Final result: all control — normal and emergency — comes from a single UART channel

9. Final System Architecture

Pin assignment

| Signal | GPIO | Dir. | Notes |
| --- | --- | --- | --- |
| UART RX | GPIO 16 | Input | Bluetooth data in (from app) |
| UART TX | GPIO 17 | Output | Bluetooth data out |
| Servo left | GPIO 18 | Output | PWM 50Hz — left door |
| Servo right | GPIO 19 | Output | PWM 50Hz — right door |
| NeoPixel DIN | GPIO 23 | Output | LED strip data (+ 470Ω resistor) |
| Motor IN1 | GPIO 25 | Output | L298N direction control |
| Motor IN2 | GPIO 26 | Output | L298N direction control |
| Motor ENA | GPIO 27 | Output | L298N enable |
| IR sensor F0 | GPIO 34 | Input | Floor 0 arrival · IRQ_FALLING · 3.3V |
| IR sensor F1 | GPIO 35 | Input | Floor 1 arrival · IRQ_FALLING · 3.3V |
| IR sensor F2 | GPIO 32 | Input | Floor 2 arrival · IRQ_FALLING · 3.3V |

Power rails

- 5 V rail → L298N VCC, both servo motors, NeoPixel strip VCC

- 3.3 V rail → all three IR sensor VCC pins

- Common GND → all components share a single ground reference

Note: IR sensors must be on 3.3 V, not 5 V, so their output signal is ESP32 GPIO-safe.

Interrupt assignments

| Component | Pin | Trigger | ISR action |
| --- | --- | --- | --- |
| IR sensor F0 | GPIO 34 | IRQ_FALLING | ir_flag[0] = True |
| IR sensor F1 | GPIO 35 | IRQ_FALLING | ir_flag[1] = True |
| IR sensor F2 | GPIO 32 | IRQ_FALLING | ir_flag[2] = True |

ISR design rules followed

- Each ISR body is a single assignment: ir_flag[n] = True. No print(), no hardware calls, no loops inside any ISR.

- ir_flag[floor] is cleared (set False) immediately before wait_for_ir() starts waiting — prevents a stale edge from a previous arrival triggering doors on the next move.

- Emergency has no ISR. It enters via UART in the main loop, which guarantees the full emergency_sequence() runs in a clean context with no re-entrancy risk.

MIT App Inventor — block summary

| Event block | Control | Action |
| --- | --- | --- |
| ListPicker1.BeforePicking | Set ListPicker1.Elements | = BluetoothClient1.AddressesAndNames |
| ListPicker1.AfterPicking | Call BluetoothClient1.Connect | address = ListPicker1.Selection |
| BtnFloor0.Click | If BluetoothClient1.IsConnected | → Call BluetoothClient1.SendText text: "0" |
| BtnFloor1.Click | If BluetoothClient1.IsConnected | → Call BluetoothClient1.SendText text: "1" |
| BtnFloor2.Click | If BluetoothClient1.IsConnected | → Call BluetoothClient1.SendText text: "2" |
| BtnEmergency.Click | If BluetoothClient1.IsConnected | → Call BluetoothClient1.SendText text: "E" |

All four command buttons use an IsConnected guard. This prevents the app from crashing or sending garbage if the user taps a button before Bluetooth pairing is complete.

Note: In the Designer view: set BtnEmergency background to red (#CC0000), increase its font size, and place it at the bottom of the screen separated from the floor buttons to avoid accidental taps.

File reference

- elevator_final.py — final MicroPython firmware for the ESP32

- Blocks implemented in MIT App Inventor 2